| ADC - Add with Carry ADD - Add without Carry ADIW - Add Immediate to Word AND - Logical AND ANDI - Logical AND with Immediateand ASR - Arithmetic Shift Right BCLR - Bit Clear in SREG BLD - Bit Load from the T Flag in SREG to a Bit in Register. BRBC - Branch if Bit in SREG is Cleared BRBS - Branch if Bit in SREG is Set BRCC - Branch if Carry Cleared BRCS - Branch if Carry Set BREAK - Break BREQ - Branch if Equal BRGE - Branch if Greater or Equal Signed) BRHC - Branch if Half Carry Flag is Cleared BRHS - Branch if Half Carry Flag is Set BRID - Branch if Global Interrupt is Disabled BRIE - Branch if Global Interrupt is Enabled BRLO - Branch if Lower (Unsigned) BRLT - Branch if Less Than (Signed) BRMI - Branch if Minus BRNE - Branch if Not Equal BRPL - Branch if Plus BRSH - Branch if Same or Higher (Unsigned) BRTC - if the T Flag is Cleared BRTS - Branch if the T Flag is Set BRVC - Branch if Overflow Cleared BRVS - Branch if Overflow Set BSET - Bit Set in SREG SEH - Set Half Carry Flag SEI - Set Global Interrupt Flag SEN - Set Negative Flag SER - Set all bits in Register SES - Set Signed Flag SET - Set T Flag SEV - Overflow Flag SEZ - Set Zero Flag SLEEP- Sleep mode SPM - Store Program Memory ST - Store Indirect From Register to data space using Index X ST (STD) - Store Indirect From Register to data space using Index Y | BST - Bit Store from Bit in Register to T Flag in SREG CALL - Long Call to a Subroutine CBI - Clear Bit in I/O Register CBR - Clear Bits in Register CLC - Clear Carry Flag CLH - Clear Half Carry Flag CLI - Clear Global Interrupt Flag CLN - Clear Negative Flag CLR - Clear Register CLS - Clear Signed Flag CLT - Clear T Flag CLV - Clear Overflow Flag CLZ - Clear Zero Flag COM- One's Complement CP- Compare CPC- Compare with Carry CPI- Compare with Immediate CPSE- Compare Skip if Equal DEC- Decrement EICALL - Extended Indirect Call to Subroutine EIJMP - Extended Indirect Jump ELPM - Extended Load Program Memory EOR - Exclusive OR FMUL- Fractional Multiply Unsigned FMULS - Fractional Multiply Signed FMULSU - Fractional Multiply Signed with Unsigned ICALL - Indirect Call to Subroutine IJMP - Indirect Jump IN - Load an I/O Location to Register INC- Increment JMP - Jump LD - Load Indirect from data space to Register using Index X LAT - Load and Toggle LAS - Load and Set ST (STD) - Store Indirect From Register to data space using Index Z STS - Store Direct to data space STS - Store Direct to SRAM SUB- Subtract without Carry SUBI- Subtract Immediate SWAP - Swap Nibbles TST- Test for Zero or Minus WDR - Watchdog Reset XCH - Exchange | LAC - Load and Clear LD (LDD)- Load Indirect from data space to Register using Index Y LD (LDD) - Load Indirect From data space to Register using Index Z LDI - Load Immediate LDS - Load Direct from data space LDS - Load Direct from SRAM LPM - Load Program Memory LSL- Logical Shift Left LSR- Logical Shift Right MOV - Copy Register MOVW - Copy Register Word MUL- Multiply Unsigned MULS - Multiply Signed MULSU - Multiply Signed with Unsigned NEG- Two's Complement NOP - No Operation OR- Logical OR ORI- Logical OR with Immediate OUT - Store Register to I/O Location POP - Pop Register from Stack PUSH - Push Register on Stack RCALL - Relative Call to Subroutine RET - Return from Subroutine RETI - Return from Interrupt RJMP- Relative Jump ROL- Rotate Left trough Carry ROR- Rotate Right through Carry SBC- Subtract with Carry SBCI- Subtract Immediate with Carry SBI - Set Bit in I/O Register SBIC - Skip if Bit in I/O Register is Cleared SBIS - Skip if Bit in I/O Register is Set SBIW - Subtract Immediate from Word SBR- Set Bits in Register SBRC - Skip if Bit in Register is Cleared SBRS - Skip if Bit in Register is Set SEC - Set Carry Flag |
| --- | --- | --- |

| PC = Size of address MDR = Size of Data memory AC = Size of Data memory IR = Size of opcode | DDRF PINF PORTG DDRG PING are in the extended IO space so you have to use LDS and STS to use them AtMega128 has 6 8-bit IO ports and 1 5-bit |
| --- | --- |
| LPM can only use Z register | MDR is the only interface to memory(therefore same width) |
| Result of MUL lower byte is in R0 and upper byte is in R1 (even if 00) | RJMP and RCALL = 16-bit. JM and CALL = 32-bit |
| AtMega128: 32 8-bit GPRs. 64 8-bit IO regs. 160 Extended 8-bit IO regs | - X is registers 27:26 - Y is registers 29:28 - Z is registers 31:30 |
| - CALL and JMP are 32-bit instructions - If working in Program Memory (DIR<<1) - Set ZL low(DIR>>1) | - INT7-4 connected to PORTE pins 7-4. - INT3-0 connected to PORTD pins 3-0. |
| CTC Mode - Load OCR0 with a value. - TCNT0 starts at 0 and counts up to OCR0 (TOP) - OCF0 is set when TCNT0 reaches OCR0 - OCRx is used to control when counting ends. - TCNTx is reset to 0 when TCNTx and OCRx match. - Need to write a 1 to TOVx and OCFx to use them again - OCO is part of a PORT so DDRB must be set to 1 (output) in order to be read from | Normal Mode - TOV0 is set when TCNT0 reaches MAX - Smaller the prescale, the higher the resolution - Counting stops when TCNTx rolls over Fast PMW Mode - Uses both OCF0 and TOV0 - Clear OC0 on output compare match, set OC0 at TOP Timers - TOIEx = Timer overflow interrupt enable (Normal mode) - OCIEx = Output compare interrupt enable (CTC mode) - CTC Mode: move value into OCR and 0 into TCNT. Now when TCNT==OCR, OCIE will be set to call the interrupt. IF the count is to be repeated then clear TCNT to 0 and set CTCx bit which clears it on compare automatically - TCNT is compared to OCR. IF match OCF is set. IF 16-bit then OCR1A sets OCF1A - TCCRx is register that customizes timer |
| In PC relative instructions like RCALL and BREQ, you have to find the address by using PC + 1 | - Must set bit in EIMSK to be detected - You know which INT was triggered by bit testing EIFR - 00 = low, 11 = rising, 10 = falling |
| USART - Start bit is always low - Stop bit is always high - Idle is high - Even Parity means P-bit is set to make number of 1’s even - By setting TXCIE and RXCIE in UCSRxB you can set interrupts. Vectors for 0 = 0x0028 and 0x0024 and for USRART1 = 0x0040 and 0x003C | USART - UMSELx = 0 = Async, 1 = Sync - UCSZx 2:0 is data frame size - UPMx 1:0 is parity == 00 none, 10 = even, 01 = odd - USBSx = 0 = one stop bit, 1 = two stop bits - TXENx = transmitter enable - UDREx = 0 = UDRx Full, 1 = UDRx Empty - TXCx = 0 = transmit incomplete, 1 = complete - RXCx = 0 = receive incomplete, 1 = complete |
| Effective address (is pointer) routes to the left of Address Adder and Target Address(is raw address) routes to the right | sbr mpr, (1<<WskrR)¦(1<<WskrL) ;setting bits in flag register to 1 since pull up resistor out EIFR, mpr ;cancel out any staged interrupts |
|  | ldi mpr, 0b01101111 ;Activate Fast PWM mode with toggle. Done by bit 3 and 6 being 1 out TCCR0, mpr ;(non-inverting), and set prescaler to 1024. Normal Mode ;later out OCR0, newSpeed |
|  | lds mpr, UCSR1A sbrs mpr, UDRE1 ;If byte is still in UDR, wait till it shifts to transmit reg ;enable the reciever and the RX Complete Interrupt Enable ldi mpr, (1<<TXEN1)¦(1<<RXEN1)¦(1<<RXCIE1) sts UCSR1B, mpr ;later lds recvdCmmd, UDR1 |